|  | - Connect VIN to 3 volts (Pin 1) – red - Connect SDA to GPIO 2 (Pin 3) - yellow - Connect SCL to GPIO 3 (Pin 5)- orange - Connect Gnd to pin 9 - brown |
| --- | --- |

Python code import board import adafruit_mlx90614 import busio as io import time import requests import random import sys import threading from shareplum import Site from shareplum import Office365 from decimal import Decimal authcookie = Office365('https://XYZ.sharepoint.com', username='XYZ@Myconsulting.com', password='XYZ').GetCookies() site = Site('https://Myconsulting.sharepoint.com/sites/MySPLISTNAME', authcookie=authcookie) set_list = site.List('MYLISTNAME') i2c = io.I2C(board.SCL,board.SDA,frequency=100000) while True: mlx = adafruit_mlx90614.MLX90614(i2c) obj_temp=mlx.object_temperature temp = '{:04.1f}'.format(obj_temp) print(temp) my_data = [{'ID': '14','temperature':temp}] set_list.UpdateListItems(data=my_data, kind='Update') time.sleep(0)